/*!

\page pccc_bersim_awgn Simulation of PCCCs in an AWGN channel

This program simulates Parallel Concatenated Convolutional Codes (PCCCs) of coding rate 1/3

using a turbo decoder with two SISO RSC modules.

Reference: S. Benedetto, D. Divsalar, G. Motorsi and F. Pollara, "A Soft-Input Soft-Output Maximum A posteriori (MAP) Module to Decode Parallel and Serial Concatenated Codes", TDA Progress Report, nov. 1996

\include pccc_bersim_awgn.cpp

When you run this program, the results (BER and EbN0_dB) are saved into pccc_bersim_awgn.it file.

Using the following MATLAB script:

\code

clear all

itload('pccc_bersim_awgn.it');

figure

semilogy(EbN0_dB, BER, 'o-')

grid on

xlabel('E_b/N_0 [dB]')

ylabel('BER')

\endcode

the results can be displayed.

Similarly, the results can be displayed using the following Python script (pyitpp, numpy and matplotlib

modules are required):

\code

#!/usr/bin/env python

from pyitpp import itload

from matplotlib.pyplot import *

out = itload('pccc_bersim_awgn.it')

semilogy(out['EbN0_dB'], out['BER'].T, 'o-')

grid()

xlabel('$E_b/N_0 [dB]$')

ylabel('$BER$')

show()

\endcode

*/